| # | Item | Basis (P13 – 60/40 Decree) | Value |
| --- | --- | --- | --- |
| 1 | Plot Area | Given | 1,313.00 m² |
| 2 | Max Ground Floor Footprint | 60% site coverage (أرضي) | 787.80 m² |
| 3 | Max Typical Upper Floor Footprint | 40% site coverage (علوي) | 525.20 m² |
| 4 | Max General Exploitation Factor (FAR = 3) | Factor 3 × plot area | 3,939.00 m² |
| 5 | Max Building Height | Art. 11 / P13 schedule | 42.00 m |
| 6 | Min Setbacks (front/side/rear) | P13 schedule | 3.00 m each |
| 7 | Ground Floor – Stores | 1 floor @ 60% footprint | 787.80 m² |
| 8 | Mezzanine over Stores (no basement storage) | 40% of ground footprint | 315.12 m² |
| 9 | Offices above Stores | 2 floors × 525.20 m² (40% footprint) | 1,050.40 m² |
| 10 | Residential Floors (full) | 3 floors × 525.20 m² | 1,575.60 m² |
| 11 | Residential – Top Floor (partial, balance of FAR) | Remaining exploitation balance | 210.08 m² |
| 12 | Total Built-Up Area (check) | Sum of rows 7–11 | 3,939.00 m² ✓ = Row 4 |

| # | Item | Basis (P13 – 60/40 Decree) | Value |
| --- | --- | --- | --- |
| 1 | Plot Area | Given | 2,277.00 m² |
| 2 | Max Ground Floor Footprint | 60% site coverage (أرضي) | 1,366.20 m² |
| 3 | Max Typical Upper Floor Footprint | 40% site coverage (علوي) | 910.80 m² |
| 4 | Max General Exploitation Factor (FAR = 3) | Factor 3 × plot area | 6,831.00 m² |
| 5 | Max Building Height | Art. 11 / P13 schedule | 42.00 m |
| 6 | Min Setbacks (front/side/rear) | P13 schedule | 3.00 m each |
| 7 | Ground Floor – Stores | 1 floor @ 60% footprint | 1,366.20 m² |
| 8 | Mezzanine over Stores (no basement storage) | 40% of ground footprint | 546.48 m² |
| 9 | Offices above Stores | 2 floors × 910.80 m² (40% footprint) | 1,821.60 m² |
| 10 | Residential Floors (full) | 3 floors × 910.80 m² | 2,732.40 m² |
| 11 | Residential – Top Floor (partial, balance of FAR) | Remaining exploitation balance | 364.32 m² |
| 12 | Total Built-Up Area (check) | Sum of rows 7–11 | 6,831.00 m² ✓ = Row 4 |

§